## Combinatorial Circuits

### 1. Definition

A combinatorial circuit is a digital logic circuit whose output depends only on the present input values.
It does not have memory elements, so it does not store past inputs.

Output = f (current inputs only)

### 2. Characteristics

- No memory elements (no flip-flops)

- No feedback paths

- Output changes immediately when input changes

- Designed using Boolean algebra and logic gates

### 3. Basic Components

Combinatorial circuits are built using:

- AND gates

- OR gates

- NOT gates

- NAND, NOR

- XOR, XNOR

### 4. Common Types of Combinatorial Circuits

#### 1. Arithmetic Circuits

- Half Adder

- Full Adder

- Subtractor

- Parallel Adder

#### 2. Data Processing Circuits

- Multiplexer (MUX)

- Demultiplexer (DEMUX)

- Encoder

- Decoder

#### 3. Code Conversion Circuits

- Binary to Gray code

- BCD to 7-segment display

#### 4. Comparator Circuits

- Magnitude comparator

### 5. Examples

#### Example 1: Half Adder

Inputs: A, B
Outputs:

- Sum = A ⊕ B

- Carry = A · B

#### Example 2: 2-to-1 Multiplexer

Output:
Y = A'S + BS

### 6. Design Procedure of Combinatorial Circuits

- Identify input and output variables

- Construct truth table

- Derive Boolean expression

- Simplify using Boolean laws or Karnaugh Map

- Draw logic diagram

### 7. Difference Between Combinational and Sequential Circuits

| Feature | Combinational | Sequential |
| --- | --- | --- |
| Memory | No | Yes |
| Feedback | No | Yes |
| Output depends on | Present input only | Present + past inputs |
| Example | Adder | Flip-flop |

## Arithmetic Circuits

Arithmetic circuits are combinational circuits used to perform mathematical operations such as addition and subtraction in digital systems.

They are fundamental components in computers, calculators, and Arithmetic Logic Units (ALUs).

## 1. Half Adder

### Definition

A Half Adder is a combinational circuit that adds two single binary digits and produces:

- Sum (S)

- Carry (C)

### Inputs

A, B

### Outputs

- Sum (S) = A ⊕ B

- Carry (C) = A · B

### Truth Table

| A | B | Sum (S) | Carry (C) |
| --- | --- | --- | --- |
| 0 | 0 | 0 | 0 |
| 0 | 1 | 1 | 0 |
| 1 | 0 | 1 | 0 |
| 1 | 1 | 0 | 1 |

### Limitation

- Cannot add carry from previous stage

- Used only for single-bit addition

## 2. Full Adder

### Definition

A Full Adder adds three binary digits:

- Two significant bits

- One carry-in (Cin)

### Inputs

A, B, Cin

### Outputs

- Sum (S) = A ⊕ B ⊕ Cin

- Carry (Cout) = AB + BCin + ACin

### Truth Table

| A | B | Cin | S | Cout |
| --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | 0 |
| 0 | 0 | 1 | 1 | 0 |
| 0 | 1 | 0 | 1 | 0 |
| 0 | 1 | 1 | 0 | 1 |
| 1 | 0 | 0 | 1 | 0 |
| 1 | 0 | 1 | 0 | 1 |
| 1 | 1 | 0 | 0 | 1 |
| 1 | 1 | 1 | 1 | 1 |

### Implementation

- Can be constructed using two half adders and one OR gate

## 3. Subtractor

A subtractor performs binary subtraction.

### (a) Half Subtractor

Subtracts two bits (A − B)

Outputs:

- Difference (D) = A ⊕ B

- Borrow (Bo) = A'B

Truth Table:

| A | B | D | Bo |
| --- | --- | --- | --- |
| 0 | 0 | 0 | 0 |
| 0 | 1 | 1 | 1 |
| 1 | 0 | 1 | 0 |
| 1 | 1 | 0 | 0 |

### (b) Full Subtractor

Subtracts three bits:

- A (minuend)

- B (subtrahend)

- Bin (borrow in)

Outputs:

- Difference (D) = A ⊕ B ⊕ Bin

- Borrow out (Bout) = A'B + B·Bin + A'·Bin

## 4. Parallel Adder

### Definition

A Parallel Adder adds multi-bit binary numbers simultaneously.

It is formed by connecting several Full Adders in series.

### Example: 4-bit Parallel Adder

- Uses 4 full adders

- Carry output of one stage connects to next stage

- Also called Ripple Carry Adder

### Working

If adding:

A = A3 A2 A1 A0
B = B3 B2 B1 B0

Each full adder adds:

- Ai

- Bi

- Carry from previous stage

Final stage produces:

- Final Sum

- Final Carry

### Limitation

- Delay occurs due to ripple carry propagation

## Summary Table

| Circuit | Inputs | Outputs | Main Function |
| --- | --- | --- | --- |
| Half Adder | 2 | Sum, Carry | Adds 2 bits |
| Full Adder | 3 | Sum, Carry | Adds 3 bits |
| Half Subtractor | 2 | Difference, Borrow | Subtracts 2 bits |
| Parallel Adder | Multiple bits | Multi-bit sum | Adds binary numbers |

GROUP WORK

In groups of 10s attempt the below questions

Scan the HANDWRITTEN work (Include on the first page all names and admission numbers of group members) and submit through Mokaya.clinton@uoeld.ac.ke at the end of the class.

## Half Adder – Application Questions

- A digital system is required to add two single-bit sensor outputs and generate a result indicating the total and whether an overflow occurred.
Explain how the appropriate arithmetic circuit can be used to implement this system. (6 Marks)

- In a voting system, two binary inputs represent “Yes” (1) and “No” (0). Design a circuit that determines the total positive votes and whether both voted “Yes.”
Identify the suitable circuit and justify your answer. (6 Marks)

- A simple binary calculator is designed to add the least significant bits of two numbers.
Draw and explain the logic circuit required. (8 Marks)

## Full Adder – Application Questions

- A microprocessor must add two binary digits together with a carry from a previous stage.
Explain how this operation is implemented using logic gates. (6 Marks)

- A digital counter system produces a carry signal when incrementing numbers.
Describe how a full adder supports multi-bit addition in such a system. (9 Marks)

- Design a logic circuit capable of adding three one-bit inputs in a control system.
Provide Boolean expressions and explain the working principle. (10 Marks)

## Subtractor – Application Questions

- An automated inventory system subtracts the number of sold items from available stock in binary form.
Explain how a subtractor circuit can be implemented to perform this operation. (6 Marks)

- A digital comparison system must determine if one binary number is smaller than another during subtraction.
Explain how borrow generation helps in this process. (9 Marks)

- In a microcontroller-based embedded system, subtraction is implemented using 2’s complement method.
Explain how adders are used to perform subtraction. (10 Marks)

## Parallel Adder – Application Questions

- A computer system is required to add two 4-bit binary numbers simultaneously.
Explain how a parallel adder performs this operation. (6 Marks)

- A digital signal processor must perform fast arithmetic operations on 8-bit numbers.
Discuss the limitation of ripple carry adders and suggest improvement techniques. (10 Marks)

- Design a 4-bit arithmetic unit capable of adding two binary numbers and explain the flow of carry signals between stages. (12 Marks)